BSA README

BSA VERSON:   bsa0106_00.40.00

CHIP:         BCM20705(usb port)

**How to setup build enviorment

Now, the code can be built on x86 and arm platform, before building the sample code you need to set ARMGCC and PATH of your bulid

system.

Ex:

export ARMGCC=arm-openwrt-linux-gcc

export PATH=[yourtoolchian]/toolchain-arm_cortex-a5+vfpv3_gcc-4.8-linaro_uClibc-0.9.33.2_eabi/bin:$PATH

**How to buid sample code

To build sample code you should build as following:

cd 3rdparty/embedded/bsa_examples/linux/build_folder/build

make CPU=platform_name

Ex: if you want to buld app_mananger for local build machine, you can do as following:

cd 3rdparty/embedded/bsa_examples/linux/libbsa/build

make

cd 3rdparty/embedded/bsa_examples/linux/app_manager/build

make

if you want to build app_manager:

cd 3rdparty/embedded/bsa_examples/linux/app_manager/build

make CPU=arm

**Drivers

btusb: 20705 driver

in 3rdparty/embedded/brcm/linux/btusb folder, please refers to Release_btusb.txt on how to build and use the driver

bthid: The BTHID driver/module allows a user mode applicatio to submit HID events to the Linux kernel. This is only used for app_hh

in 3rdparty/embedded/brcm/linux/bthid folder, please refers to Release_bthid.txt on how to build and use the driver.

**Application Demo List

release_notes/bsa_examples/Release_app_xx.txt

app_hh -- HH (HID Host): Used to connect to HID Devices (Mouse, Keyboard, Remote Control,)

app_hd -- HD(HID Device): To act HID device

app_av -- AV (Audio/Video): Used to stream audio to stereo headset

app_avk -- AVK (Audio/Video Sink): To act like a stereo headset

app_ag -- HS/HF –AG (Audio Gateway): Used in a phone or device connected to network

app_hs -- HS/HF –HS (HeadSet/HandsFree): To act like a mono headset (used by cellular)

app_fts -- FTP Server -- FTS (File Transfer Server): Used by remote devices (cellular, PC) to access files/folders.

app_ftc -- FTP Client

app_ops -- OPP Server -- OPS Object Push Server : Used by remote devices to push/pull files (e.g. business card)

app_opc -- OPP Client

app_pbs -- PBS (Phone Book Server): Used by remote devices to access local phone book.

app_pbc -- Phone Book Profile Client

app_pam -- Personal Area Networking Profile (PAN)

app_hl -- HDP (Health Device Profile):  Used for exchange of medical device data

app_mce – MAP(Message Access Profile) client

app_3d – 3D Synchronization Profile

app_tm – Test Mode, for RF test

app_dg -- SPP (Serial Port Profile):  Used for wireless replacement of serial cable

app_ble -- GATT

app_ble_cscc – BLE CSC(Cycling speed and cadence) controller

app_ble_hrc -- BLE Heart Rate Controller

app_ble_pm -- BLE Proximity Monitor

app_ble_rscc -- BLE RSC(Running speed and cadence) controller

app_hogp -- HOGP host

**How to run the application

In 3rdparty/embedded/bsa_examples/linux/app_xx/build/Release_app_xx.txt, you can find the notes of sample applicattion.

To run the sample application, You need to make sure the following step:

1. 20705 dongle is connected with you system

2. insmod btusb. btusb driver is loaded accodrding to Release_btusb.txt

insmod /udata/nfs/btusb.ko

mknod /dev/btusb0 c 180 194

chmod 666 /dev/btusb0

3. bsa_server is released in binary, please check the bsa_server for different platform

bsa_server stored path:  3rdparty/embedded/bsa_examples/linux/server/build

4. bsa_server is started as following:

./bsa_server -d /dev/btusb0 -p your_patchram_path/patchram_file

And you can use ./bsa_server --help to check the option that can be used for bsa_server.

-d para:  uart -- /dev/ttyXX   usb -- /dev/btusb0

If you use uart port, you can set baudrate by ‘-r’.

./bsa_server -d /dev/ttyXX -p xx.hcd -r 10

-d device         set the hci device interface (e.g. /dev/ttyHS0)

-p patchfile      force the patchfile (RAM) to use

-r baudrate       set baud rate for opening port

Baud rate codes:

0  = 300 baud

1  = 600

2  = 1200

3  = 2400

4  = 9600

5  = 19200

6  = 57600

7  = 115200

8  = 230400

9  = 460800

10 = 921600

11 = 1M

12 = 1.5M

13 = 2M

14 = 3M

15 = 4M

5. run ./app_manager

6. run sample application that you need, and you can try the sample application

**How to catch log

For issue tracking, bsa log and btsnoop are necessary.

Ex.  ./bsa_server -d /dev/btusb0 -p xx.hcd -all=5 -b btsnoop.cfa > bsa.log

1. bsa log

You call use “-all=#” to set trace level to # for all layers.

Usually you can set “all=5” to catch issue trace for developing.

And you can set “all=0” to disable all trace for publishing.

2.  btsnoop

You can use “-b file”  to specify the BT Snoop file to save serial packets.

** Detailed Example for Application running

Chip: Based on 20705 usb port

AV Headset : Motorola Elite Flip

HID device: Microsoft Sculpt Mobile Keyboard

BLE HID device: Xiaomi Remote Controller

- AV -- connect stereo headset and play music

- First pair and play:

Preparatory work:

- put music to Music folder path:  ./test_files/av

- Open Headset, entry Bluetooth pair mode.

Start:

- Run ./bsa_server -d /dev/btusb0 -p patchram.hcd

- When bsa_server started successfully, run ./app_manager, ./app_av in sequence.

- app_av menu:  input  2  (Start Discovery)

- input  6  (start connect)

- input  1

- input  0

- app_manager  input  10   (accept pair)

- you can hear the headset hint connected, and find:

app_manager:

app_av:

- Play File

input 10 to play specified music OR input 11 to play music list in folder.

If input 10, and then select specified music, as below:

- Reconnect Headset

If you disconnect Headset and want to reconnect:

- run ./bsa_server -d /dev/btusb0 -p patchram.hcd (if not running)

- run ./app_manager,  ./app_av (if not running)

- open BT headset

2.  Audio gateway(connect to headset)

a. Put "sco_ag_out.wav"  into ./test_files/ag

b. Run ./bsa_server -d /dev/btusb0 -p patchram.hcd

c. When bsa_server started successfully, run ./app_manager, ./app_ag in sequence.

d. app_ag menu(See the bold red part):

./app_ag

BSA_trace 1@ 01/01 23h:14m:13s:748ms: BSA_MgtOpenInit

BSA_trace 2@ 01/01 23h:14m:13s:748ms: BSA_MgtOpen (./)

Created thread with thread_id=  df18c700

BSA_trace 3@ 01/01 23h:14m:13s:748ms: bsa_cl_mgt_init

BSA_trace 4@ 01/01 23h:14m:13s:748ms: UIPC_Init

BSA_trace 5@ 01/01 23h:14m:13s:748ms: UIPC_Open ChId:3

DEBUG: app_ag_enable: Entering

BSA_trace 6@ 01/01 23h:14m:13s:748ms: BSA_AgEnableInit

BSA_trace 7@ 01/01 23h:14m:13s:748ms: BSA_AgEnable

DEBUG: app_ag_register: Entering

BSA_trace 8@ 01/01 23h:14m:13s:749ms: BSA_AgRegisterInit

BSA_trace 9@ 01/01 23h:14m:13s:749ms: BSA_AgRegister

DEBUG: app_ag_register: Register complete handle 1, status 0/success

Bluetooth Headset/HandsFree Main menu:

1  => Discover Headset

2  => Enable

3  => Disable

4  => Register

5  => Deregister

6  => Connect

7  => Disconnect

8  => Open audio

9  => Close audio

10  => Record audio file

11  => Stop record audio file

12  => Play audio file

13  => Stop Playing audio file

14  => Display SCO route config

15  => Indicate incoming call

16  => PickUp call

17  => HangUp call

99  => Quit

Select action => 1

Start COD filtered Discovery

Look for COD service:2000 major:4 minor:0

BSA_trace 10@ 01/01 23h:14m:15s:804ms: BSA_DiscStartInit

COD: 0x20 0x04 0x00

COD Mask: 0x20 0x1f 0x00

BSA_trace 11@ 01/01 23h:14m:15s:804ms: BSA_DiscStart

Bluetooth Headset/HandsFree Main menu:

1  => Discover Headset

2  => Enable

3  => Disable

4  => Register

5  => Deregister

6  => Connect

7  => Disconnect

8  => Open audio

9  => Close audio

10  => Record audio file

11  => Stop record audio file

12  => Play audio file

13  => Stop Playing audio file

14  => Display SCO route config

15  => Indicate incoming call

16  => PickUp call

17  => HangUp call

99  => Quit

Select action => New Discovered device:0

Bdaddr:00:0c:8a:3c:13:2d

Name:Bose BT2 01.05.00

ClassOfDevice:24:04:04 => Audio/Video

Services:0x00000000 ()

Rssi:-51

DeviceType:BR/EDR InquiryType:BR AddressType:Public

Extended Information:

ShortName: Bose BT2 01

TxPower:4 dB

Complete Service [UUID16]:

0x111E [Handsfree]

0x1108 [Headset]

0x110D [Advanced Audio Distribution]

0x110B [Audio Sink]

Discovery complete

6

DEBUG: app_ag_open: Entering

Bluetooth AG menu:

0 Device from XML database (already paired)

1 Device found in last discovery

Select source => 1

Dev:0

Bdaddr:00:0c:8a:3c:13:2d

Name:Bose BT2 01.05.00

ClassOfDevice:24:04:04 => Audio/Video

Rssi:-51

Select device => 0

BSA_trace 12@ 01/01 23h:14m:25s:140ms: BSA_AgOpenInit

BSA_trace 13@ 01/01 23h:14m:25s:140ms: BSA_AgOpen

BSA_trace 14@ 01/01 23h:14m:25s:140ms: BSA_AgOpen BDA: 00:0c:8a:3c:13:2d

Bluetooth Headset/HandsFree Main menu:

1  => Discover Headset

2  => Enable

3  => Disable

4  => Register

5  => Deregister

6  => Connect

7  => Disconnect

8  => Open audio

9  => Close audio

10  => Record audio file

11  => Stop record audio file

12  => Play audio file

13  => Stop Playing audio file

14  => Display SCO route config

15  => Indicate incoming call

16  => PickUp call

17  => HangUp call

99  => Quit

Select action => BSA_trace 15@ 01/01 23h:14m:27s:956ms: bsa_ag_event_hdlr event:0

DEBUG: app_ag_cback: event:0 for handle: 1, index: 0

DEBUG: app_ag_cback: BSA_AG_OPEN_EVT: s=0(success)

DEBUG: app_ag_cback: 00:0c:8a:3c:13:2d

BSA_trace 16@ 01/01 23h:14m:28s:186ms: bsa_ag_event_hdlr event:13

DEBUG: app_ag_cback: event:13 for handle: 1, index: 0

DEBUG: app_ag_cback: BSA_AG_AT_CIND_EVT

BSA_trace 17@ 01/01 23h:14m:28s:186ms: BSA_AgResult

BSA_trace 18@ 01/01 23h:14m:28s:302ms: bsa_ag_event_hdlr event:2

DEBUG: app_ag_cback: event:2 for handle: 1, index: 0

DEBUG: app_ag_cback: BSA_AG_CONN_EVT: 00:0c:8a:3c:13:2d

BSA_trace 19@ 01/01 23h:14m:28s:369ms: bsa_ag_event_hdlr event:18

DEBUG: app_ag_cback: event:18 for handle: 1, index: 0

DEBUG: app_ag_cback: BSA_AG_AT_NREC_EVT

BSA_trace 20@ 01/01 23h:14m:28s:439ms: bsa_ag_event_hdlr event:6

DEBUG: app_ag_cback: event:6 for handle: 1, index: 0

DEBUG: app_ag_cback: BSA_AG_SPK_EVT 12

BSA_trace 21@ 01/01 23h:14m:28s:590ms: bsa_ag_event_hdlr event:21

DEBUG: app_ag_cback: event:21 for handle: 1, index: 0

DEBUG: app_ag_cback: BSA_AG_AT_CLCC_EVT

BSA_trace 22@ 01/01 23h:14m:28s:590ms: BSA_AgResult

8

DEBUG: app_ag_open_audio: Entering

BSA_trace 23@ 01/01 23h:14m:32s:628ms: BSA_AgAudioOpenInit

BSA_trace 24@ 01/01 23h:14m:32s:628ms: BSA_AgAudioOpen

Bluetooth Headset/HandsFree Main menu:

1  => Discover Headset

2  => Enable

3  => Disable

4  => Register

5  => Deregister

6  => Connect

7  => Disconnect

8  => Open audio

9  => Close audio

10  => Record audio file

11  => Stop record audio file

12  => Play audio file

13  => Stop Playing audio file

14  => Display SCO route config

15  => Indicate incoming call

16  => PickUp call

17  => HangUp call

99  => Quit

Select action => BSA_trace 25@ 01/01 23h:14m:32s:678ms: bsa_ag_event_hdlr event:3

DEBUG: app_ag_cback: event:3 for handle: 1, index: 0

DEBUG: app_ag_cback: BSA_AG_AUDIO_OPEN_EVT: s=0(success)

BSA_trace 26@ 01/01 23h:14m:32s:678ms: UIPC_Open ChId:9

BSA_trace 27@ 01/01 23h:14m:32s:678ms: uipc_rb_open name:sco-tx-rb id:9 size:2401 flags:x2

BSA_trace 28@ 01/01 23h:14m:32s:678ms: uipc_rb_open: Size = 2401

BSA_trace 29@ 01/01 23h:14m:32s:679ms: uipc_rb_open: Buffer = 0x7f92dfb9d010

BSA_trace 30@ 01/01 23h:14m:32s:679ms: uipc_rb_open: semid = 720896

BSA_trace 31@ 01/01 23h:14m:32s:679ms: uipc_rb_open: wr_lockid = 753665

BSA_trace 32@ 01/01 23h:14m:32s:679ms: uipc_rb_open: wr_lockid.val = 2400

BSA_trace 33@ 01/01 23h:14m:32s:679ms: uipc_cl_sco_channel_open SCO channels opened

BSA_trace 34@ 01/01 23h:14m:32s:679ms: uipc_rb_open name:sco-rx-rb id:10 size:2401 flags:x1

BSA_trace 35@ 01/01 23h:14m:32s:679ms: uipc_rb_open: Size = 2401

BSA_trace 36@ 01/01 23h:14m:32s:679ms: uipc_rb_open: Buffer = 0x7f92dfb9c010

BSA_trace 37@ 01/01 23h:14m:32s:679ms: uipc_rb_open: semid = 819203

BSA_trace 38@ 01/01 23h:14m:32s:679ms: uipc_rb_open: wr_lockid = 851972

BSA_trace 39@ 01/01 23h:14m:32s:679ms: uipc_rb_open: wr_lockid.val = 2400

BSA_trace 40@ 01/01 23h:14m:32s:679ms: uipc_cl_sco_channel_open SCO channels opened

BSA_trace 41@ 01/01 23h:14m:32s:679ms: UIPC_Ioctl ChId:9 Request:UIPC_REG_CBACK (5)

BSA_trace 42@ 01/01 23h:14m:32s:679ms: uipc_cl_sco_ioctl UIPC_REG_CBACK 0

BSA_trace 43@ 01/01 23h:14m:32s:679ms: uipc_cl_sco_read_task p_cback 0x4023b0 , arg 0x4023b0

12

Bluetooth Headset/HandsFree Main menu:

1  => Discover Headset

2  => Enable

3  => Disable

4  => Register

5  => Deregister

6  => Connect

7  => Disconnect

8  => Open audio

9  => Close audio

10  => Record audio file

11  => Stop record audio file

12  => Play audio file

13  => Stop Playing audio file

14  => Display SCO route config

15  => Indicate incoming call

16  => PickUp call

17  => HangUp call

99  => Quit

Select action => DEBUG: app_ag_play_file_thread: Entering

3. HID Device

Preparatory work:

MS Keyboard entry Bluetooth pair mode.

Start:

- Run ./bsa_server -d /dev/btusb0 -p patchram.hcd

- When bsa_server started successfully, run ./app_manager, ./app_hh in sequence.

- app_hh menu:  input  2  (COD Discovery (Peripheral Dev))

- input  6   (HID Connect (Report mode))

- input 1 , and then input 0 to select MS Keyboard

- input 0,  connect successfully. (Security select depends on HID device)

if you input key in keyboard, you can find data display on app_hh:

For example, press “ESC”

4. BLE HID device:

Preparatory work:

Xiaomi BLE remote controller entry Bluetooth pair mode.

Start:

a.  Run ./bsa_server -d /dev/btusb0 -p patchram.hcd

b.  When bsa_server started successfully, run ./app_manager, ./app_ble, ./app_hh in sequence(Or just run app_hogp)

c. input 4  (BLE HID deivce Discovery)

d. input  6   (HID Connect (Report mode))

e. input 1 , and then input 0 to select Xiaomi RC

f. input 0, connect successfully. (Security select depends on HID device)

At app_hh side, the display about connected successfully and pressed key is similar with BR/EDR HID devices.